Data-driven location–allocation for clean cooking LPG supply chains: A mixed-integer programming approach for Rwanda [1]

- We formulate a hierarchical location–allocation problem as a Mixed-Integer Linear Program (MILP), aiming to minimize total system cost across the full supply chain — from importation to retail distribution. […] MILP formulation capable of evaluating infrastructure–transportation trade-offs across an entire country.

- we adopt two complementary strategies: (i) a time-aggregated formulation that assumes stable seasonal demand, and (ii) a spatial aggregation method based on agglomerative hierarchical clustering, which reduces the number of retailer decision points by locating them at distance-constrained geomedian points of rooftop clusters.

- We compare this clustering-based approach with a benchmark that places retailers at village centroids, evaluating differences in cost and spatial fairness. In this study, we use the term spatial fairness to refer to the equitable spatial distribution of access to LPG retailers, ensuring that households do not face disproportionately long travel distances to obtain fuel. Access proximity is an important fairness criterion in Rwanda, where transportation constraints and income limitations can create barriers to LPG adoption in remote communities. By explicitly incorporating a distance threshold into the retailer siting approach, our framework seeks to promote more balanced access across both urban and rural areas

- Recent advances in data-driven spatial optimization for energy and health systems demonstrate the value of detailed, real-world datasets for planning  but these methods have not previously been applied to LPG distribution networks.

- the Rwanda National LPG Master Plan indicates that marketers primarily favor importing via Tanzania using their own bulk trucks (Ntamwiza, 2020, Rwanda Ministry of Infrastructure, 2021). Upon arrival, LPG is stored in a central depot, which functions as a bulk storage hub. From there, it is transported to bottling stations using tankers. This bulk transportation phase involves moving large quantities of LPG in liquid form.

- households are assumed to travel to the retailer themselves

- To determine [future] retailer locations, we explore two approaches: (1) placing a retailer at each village centroid, and (2) applying a heuristic based on agglomerative hierarchical clustering [= meaning: place the retailer where the most buildings are, ignoring the real centroid of the village] [for our purpose it would make more sense to use directly the population density…]

- Warehouse locations are determined by selecting the geographic center of each of Rwanda’s 30 districts, resulting in 30 candidate warehouse sites

- Candidate bottling station sites are selected based on guidance from the Rwanda National LPG Master Plan [7 sites selected]

- LPG is imported into Rwanda primarily from Kenya and Tanzania, with typical costs ranging from $765 to $807 per metric ton. For modeling purposes, we use the average value of $786 per metric ton, based on current estimates (Serra Llavona, 2023). A regulatory import license — valid for five years — is also required at a cost of $1976.

- Transportation is modeled across three vehicle types — tankers, trucks, and vans — used respectively for bulk, primary, and secondary distribution. Each vehicle type is characterized by its acquisition cost, fuel consumption, capacity, operating costs, and license requirements. Apparently the source for these parameters is Serra Llavona (2023) [studio non più reperibile…]

Capacity of a van (kg) - 3240

Capacity of a truck (kg) - 9996

Capacity of a tanker (kg) - 24,000

Speed of a van (km/h) - 15

Speed of a truck (km/h) - 30

Speed of a tanker (km/h) - 20

Lifetime of a van (year) - 10

Lifetime of a truck (year) - 15

Lifetime of a tanker (year) - 35

Lifetime of a cylinder (year) - 20

Van license validity (year) - 5

Truck license validity (year) - 5

Tanker license validity (year) - 5

Bottling capacity of a bottling station (kg) - 35,000,000

Capacity of a warehouse (kg) - 100,000,000

Bulk inventory capacity of a bottling station(kg) - 100,000,000

Cylinder inventory capacity of a bottling station - 100,000,000 (cylinder)

Fixed cost of a warehouse ($/year) - 154,800

Overnight cost of a warehouse ($) - 54,039.992

Fixed cost of a bottling station ($/year) - 348,400

Overnight cost of a bottling station ($) - 7,000,000

Variable cost of a bottling station ($/kg) - 0.035

Lifetime of a warehouse (year) - 15

Lifetime of a bottling station (year) - 15

Lifetime of the depot (year) - 50

Fixed cost of a van ($/year) [= cost to keep the van operable] - 127% of variable cost

Overnight cost of a van ($) [= investment cost] - 57,818.581

Variable cost of a van ($/km) - 0.437

Fixed cost of a truck ($/year) - 127% of variable cost

Overnight cost of a truck ($) - 170,200

Variable cost of a truck ($/km) - 0.540

Fixed cost of a tanker ($/year) - 127% of variable cost

Overnight cost of a tanker ($) - 197,672.677

Variable cost of a tanker ($/km) - 0.635

Cost of a 12-kg cylinder ($) - 12.10

Labor cost (%) - 1.18

Annual salary ($) - 2604

Daily working hours (h) - 8

Van license cost ($) - 418

Truck license cost ($) - 418

Tanker license cost ($) - 418

Warehouse wholesale license cost ($) - 836

Cylinder requalification cost ($/cylinder) - 7.17

Bottling station installation fee ($) - 760

Depot installation fee ($) - 760

Bottling station operating license cost ($) - 3952

Depot operating license cost ($) - 3952

Importation license cost ($) – 1976

Import license validity (year) - 5

Warehouse wholesale license validity (year) - 5

Bottling station operating license validity (year) - 3

Depot operating license validity (year) - 3

Cylinder requalification validity (year) - 10

Discount factor of a warehouse - 0.131

Discount factor of a bottling station - 0.131

Discount factor of the depot - 0.101

Discount factor of a van - 0.163

Discount factor of a truck - 0.131

Discount factor of a tanker - 0.104

Discount factor of a cylinder - 0.117

Discount factor of a van license - 0.264

Discount factor of a truck license - 0.263

Discount factor of a tanker license - 0.264

Discount factor of a warehouse wholesale license - 0.264

Discount factor of a cylinder requalification - 0.163

Discount factor of a bottling station operating license - 0.402

Discount factor of a depot operating license - 0.402

Discount factor of import license - 0.264

- The clustering model successfully ensured that all simulated households were within a 1-mile walking radius of their assigned retailer

- When testing projected demand increases from 2025 to 2030, the system initially adapts by expanding the capacity of existing bottling stations. However, long-term, significant demand growth eventually justifies a shift in the supply chain layout

- A sparse infrastructure network heavily inflates transportation costs due to longer delivery routes and higher vehicle requirements. Conversely, building a denser network of local facilities lowers transport costs but requires much higher upfront capital investments

LPG supply chain is, in technical terms, a “multi-echelon distribution system” (echelon = node)

The problem of deciding the supply points and associating the people to the supply points is, in technical terms, a “location-allocation problem”